LogAbstract MRV Requirements

My TOWT Software

# Software Inputs

## Parameters to set

- Avg MDO Density (normally between 0,82 and 0,87)

- IMO per vessel (to be included if it is not already provide in another software module)

- MDO admissible deviation (first fixed to 2mt but editable by tech/data users)

## Inputs

- Event type

  - Departure

  - Arrival

  - At sea (to open or close the reporting year)

  - Begin Anchoring/Drifting

  - End Anchoring/Drifting

- Data to collect from each event :

  - Port Main Engine DO Counter

  - Starboard Main Engine DO Counter

  - FWD Generator DO Counter

  - AFT Generator DO Counter

  - ROB in metric tons

  - Cargo MRV in metric tons (displacement – lightship)

- For departure event :

  - Bunkering quantity in metric tons

  - Bunkering date

# Data Quality Evaluation

  - DO Counters should increase with time. DO Counter is the running fuel consumption of the engines from the beginning of ship exploitation to the current event

  - Declared ROB should be consistent according to the DO Counters (cf. MDO consumption & ROB Calculation)

  - A warning should be appear if the deviation between declared ROB et calculated ROB is less than the MDO admissible deviation parameter

  - An issue should be launch if the deviation is higher than the MDO admissible deviation parameter. System should require a data update or send a notification to the quality/tech/reporting department.

  - Cargo from departure to arrival and during anchoring time should be the same

# Calculations & additional data sources

## MDO consumption & ROB

Remaining On Board MDO

Current Event ROB=Previous event ROB+Bunkering-Total Consumption

Total Consumption

Total Consumption=ME Consumption+AE Consuption

Main Engine Consumption

ME Consumption=Port Main Engine Consumption+Starboard Main Engine COnsumption

Auxiliar Engine Consumption

AE Consumption=FWD Generator Consumption+AFT Generator Consumption

For each engine, consumption can be calculated as follows :

Engine Consumption [l]=Current event counter-Previous event counter

Engine Consumption mt=Engine Consumption l×Avg MDO Density ×0,001

Please note that for MRV reporting consumption and ROB should be entered in metric tons.

## Other fields source & calculations

| Field | Source/Calculation |
| --- | --- |
| IMO | According to vessel settings |
| Date_UTC | According to vessel real schedule/tracking data or anchoring timestamp |
| Time_UTC | According to vessel real schedule/tracking data or anchoring timestamp |
| Voyage_From | According to vessel real schedule/tracking data |
| Voyage_To | According to vessel real schedule/tracking data |
| Event | Input |
| Time_Since_Previous_Report | Current Event Timestamp – Previous Event Timestamp |
| Distance | For departure, it should be normally 0 |
|  | For begin of anchoring, distance from departure to anchoring according to tracking data & event timestamp |
|  | For end of anchoring, it should be normally 0 |
|  | For arrival reports it should be total trip distance. Nevertheless, if there’s an anchoring during voyage, distance from the end of anchoring to arrival according to tracking data & event timestamp |
| Cargo_Mt | Cargo during trip (departure, begin/end of anchoring) should be normally the same |
| Latitude_Degree | According to tracking data & event timestamp |
| Latitude_Minutes | According to tracking data & event timestamp |
| Latitude_North_South | According to tracking data & event timestamp |
| Longitude_Degree | According to tracking data & event timestamp |
| Longitude_Minutes | According to tracking data & event timestamp |
| Longitude_East_West | According to tracking data & event timestamp |

# Carbon report

The system should also provide a carbon report for port calls and for each trip. This carbon report can be requested by authorities in case of control. It can be an Excel file or PDF file. If there are discrepancies within the provide data, users should correct data before the system can create the report.

# MRV Compatible Export

MRV Compatible export can be an excel or csv file with the following fields and format :

| Field | Format |
| --- | --- |
| IMO | Integer |
| Date_UTC | YYYY-MM-DD |
| Time_UTC | HH:MM |
| Voyage_From | UNCODE |
| Voyage_To | UNCODE |
| Event | As defined in the inputs section |
| Time_Since_Previous_Report | Integer (hours) |
| Distance | Integer (nautical miles) |
| Cargo_Mt | Float (metric tons) |
| ME_Consumption_MDO | Float (metric tons) |
| AE_Consumption_MDO | Float (metric tons) |
| MDO_ROB | Float (metric tons) |
| Latitude_Degree | Integer |
| Latitude_Minutes | Integer |
| Latitude_North_South | Char (“N” or “S”) |
| Longitude_Degree | Integer |
| Longitude_Minutes | Integer |
| Longitude_East_West | Char (“E” or “W”) |

# MRV Data Quality (Additional Information)

- Please note that DNV Veracity Software algorithm compares:

  - The declared timestamp according to AIS information

  - The declared distance with calculated distance according to AIS

  - The departure and arrival ports according to AIS

  - The time from previous report according to declared timestamps

  - Declared coordinates according to AIS

  - The consumption and ROB consistency with bunker operations

  - The consistency between consumption and ROB declared

  - Declared consumption with technical engine specifications and estimated consumption

- Please note that current carbon reports are manually filled, so 2025 reporting had many deviations compared to calculated values.

- My TOWT calculated values should reduce discrepancies between declared data